# Rendering Profiling Pack

# Scene Setup

- Alpha-Tested Cypress Tree (from the Unity URP Terrain Demo pack)

- Opaque power boxes (from the TirgamesAssets Factory pack)

- Lights:

  - Direction Light

  - 2-point lights

  - Spotlight

# Forward & Deferred Rendering

|  | Forward | Deferred |
| --- | --- | --- |
| Inspector |  |  |
| Frame Debugger |  |  |
| Game View |  |  |
| GPU Profiling |  |  |
| Overdraw |  |  |

In this controlled scene (1 opaque + 1 alpha-tested foliage + 4 lights), deferred rendering was slightly faster than forward, while draw calls remained nearly identical.

The cost came from foliage overlap overdraw, render path changed pass structure more than geometry workload.

In conclusion for this test, path choice had a modest impact while foliage representation is likely the stronger optimization lever in Step 3.”

## Record

| Metric | Forward | Deferred | Notes |
| --- | --- | --- | --- |
| GPU Frame Time (ms) | 3.10 ms | 2.64 ms | Deferred faster by 0.46 ms (14.8%) |
| CPU Frame Time (ms) | 4.28 ms | 4.54 ms | Similar overall with minor variation |
| Draw Calls | 62 | 63 | Equivalent in this setup |
| Dominant GPU Passes | SSAO (1.212), DrawOpaqueObjects (0.932), RG_UberPost (0.281), Bloom (0.192), RG_FinalBlit (0.190), DrawDepthNormalPrepass (0.171) | SSAO (0.699), DrawGBuffer (0.538), RenderDeferredLighting (0.386), CopyDepth (0.284), RG_UberPost (0.260), Bloom (0.183), RG_FinalBlit (0.176) | Forward concentrates lighting/object shading in opaque pass. Deferred splits into GBuffer + lighting pass. |
| Frame Debugger RenderGraph Events | ~45 | ~44 | Near-identical complexity |
| Overdraw (All mode) | High overlap at foliage center (yellow/red hotspots), blue background | Similar foliage hotspot pattern | Overdraw source is foliage overlap, not render-path change |
| Visual Result | Comparable look | Slight perceived detail difference | Likely AA/path/post differences; not major correctness issue |

# Foliage

Dense micro-triangles

LOD-simplified